TABLE OF CONTENTS

| 1. | Purpose |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. | Scope and Application |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 3. | Responsibilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | a. | Program Administrator |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | b. | Supervisor |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
|  | c. | Employees |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| 4. | Program Elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  | a. | Hazard Assessment and Respirator Selection |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  | b. | Voluntary Respirator Use |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  | c. | Fit Testing |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
|  | d. | Respirator Use - Responsibilities for Employees |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
|  | e. | Emergency Procedures (respirator malfunction) |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
|  | f. | Cleaning, Maintenance, and Training |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| 5. | Program Evaluation |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |
| 6. | Documentation and Recordkeeping |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |
|  | Appendix D |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |
|  | Exhibit 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |

| - Purpose |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Correctional Healthcare has determined that certain employees in our facility may become exposed to airborne transmissible diseases, such as COVID-19. The purpose of this Respiratory Protection Program (“Respiratory Protection Program” or “Program”) is to ensure that all employees are protected from exposure to these airborne transmissible diseases through appropriate use of respiratory protection. Engineering and administrative controls are also used to protect employees; however, these controls are not always feasible. In these situations, respirator use may be needed. |  |  |  |  |  |  |  |
| - Scope and Application |  |  |  |  |  |  |  |
| This Program applies to all employees who are required to wear respirators when performing their job duties. It also applies to employees who wish to voluntarily wear a respirator, except for those employees who voluntarily choose to wear a filtering facepiece respirator (“FFR”). In all situations of voluntary use of any respirator, Alternative Correctional Healthcare will ensure that a copy of Appendix D of this Program is provided to the employee if requested. |  |  |  |  |  |  |  |
| - Responsibilities |  |  |  |  |  |  |  |
|  | - Program Administrator |  |  |  |  |  |  |
|  |  | The Program Administrator is responsible for administering the Respiratory Protection Program. Responsibilities of the Program Administrator include: - Identifying situations requiring a mandatory use of respirators. - Selection of respiratory protection options that meet guidelines for the situation. - Monitoring respirators use to ensure that respirators are used in accordance with their certifications. - Arranging for and/or conducting training. - Ensuring proper storage, cleaning, inspections, and maintenance of respiratory stock and testing equipment. - Ensuring the conduct of fit testing. - Administering the medical evaluation and surveillance program. - Maintaining records required by the Program. - Evaluating the Program. - Updating the Program, as needed. The Program Administrator is Sallie Albert and can be contacted at sallie.albert@alternativecorrectionalhealthcare.com. |  |  |  |  |  |
|  |  |  | Page 1 |  |  |  |  |
|  | - Supervisor |  |  |  |  |  |  |
|  |  | Supervisors are responsible for ensuring that the Respiratory Protection Program is implemented. In addition to being knowledgeable about the Program requirements for their own protection, supervisors must also ensure that the Program is understood and followed by employees. Responsibilities of the supervisor include: - Ensuring that employees under their supervision (including new hires) have received appropriate training, and annual medical evaluation and fit test if duties involve use of a respirator. - Ensuring the availability of appropriate respirators. - Being aware of tasks requiring the use of respiratory protection. - Enforcing the proper use of respiratory protection when necessary. - Ensuring that respirators are properly cleaned, maintained, inspected, and stored according to the Respiratory Protection Program. - Ensuring that respirators fit well and do not cause discomfort. - Coordinating with the Program Administrator on how to address hazards or other concerns regarding the Program. |  |  |  |  |  |
|  | - Employees |  |  |  |  |  |  |
|  |  | Employees have the responsibility to: - Wear their respirators when and where required and in the manner in which they were trained for donning and doffing. - Care for and maintain their respirators as instructed, and store them in a clean, sanitary location when reuse protocols are in effect. - Inform their supervisor if the respirator no longer fits and request a new one. - Inform their supervisor or the Program Administrator of any respiratory hazards they feel are not adequately addressed in the workplace and of any other concerns that they have - regarding the Program. - Inform their supervisor of need for a medical re-evaluation. |  |  |  |  |  |
|  |  | Nurse practitioner Mental health provider RN LPN Med tech Medical Secretary |  |  |  |  |  |
|  |  | Page 2 |  |  |  |  |  |
| - Program Elements |  |  |  |  |  |  |  |
|  | - Hazard Assessment and Respirator Selection |  |  |  |  |  |  |
|  |  | The Program Administrator will select respirators to be used based on the airborne transmissible disease hazards to which employees are exposed and in accordance with all applicable OSHA standards. The Program Administrator will conduct a hazard evaluation and document the evaluation. The hazard evaluation will involve a review of job tasks to determine where potential exposures may occur with employees. The Program Administrator will revise and update the hazard assessment as needed. If an employee feels that respiratory protection is needed during a particular activity, he/she is to contact his or her supervisor or the Program Administrator. The Program Administrator will then: - Evaluate the potential hazard, arranging for outside evaluation as necessary. - Communicate the results of that assessment back to the employees. If it is determined that respiratory protection is necessary, all other elements of this Program will be in effect for those tasks, and this Program will be updated accordingly. |  |  |  |  |  |
|  |  | All of the above This will be determined at to which point the outbreak is centered in the facility. |  |  |  |  |  |
|  | - Voluntary Respirator Use |  |  |  |  |  |  |
|  |  | Alternative Correctional Healthcare also allows for voluntary use of respirators under all circumstances. Under the voluntary Respirator Use Program, Alternative Correctional Healthcare will: - Authorize voluntary use of respiratory protective equipment as requested by employees, depending on specific workplace conditions and the results of medical evaluations. - Provide all employees who voluntarily choose to wear a respirator with a copy of Appendix D as specified by OSHA’s Respiratory Protection Standard (29 CFR 1910.134). Appendix D details the requirements for voluntary use of respirators by workers and is appended to this Program. |  |  |  |  |  |
|  |  |  | Page 3 |  |  |  |  |
|  | - Fit Testing |  |  |  |  |  |  |
|  |  | Employees who are required to wear tight-fitting respirators will be fit tested: - Prior to being allowed to wear any tight-fitting respirator. - Annually, and - When there are changes in the employee’s physical condition that could affect respirator fit (e.g., obvious change in body weight, facial scarring, etc.). Employees will be fit tested with the make, model, and size of respirator that they will actually wear. The Program Administrator will ensure fit tests are conducted in accordance with any appropriate and OSHA-approved protocol from Appendix A of the Respiratory Protection Standard: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.134AppA. |  |  |  |  |  |
|  |  | Within the first 2 weeks of hire every employee will be fit tested by the partner facility. If the partner facility is unable to fit test, the employee will be directed to OHA for testing. |  |  |  |  |  |
|  | - Respirator Use - Responsibilities for Employees |  |  |  |  |  |  |
|  |  | All employees wearing a respirator are responsible for and agree to: - Use respirators under conditions specified by this Program, and in accordance with their training. The respirator must not be used in a manner for which it is not certified by NIOSH or by its manufacturer. - Conduct user seal checks each time the respirator is worn, in accordance with the Respiratory Protection Standard: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.134AppB1. - Not wear tight-fitting respirators if they have any condition, such as facial scars, facial hair, or missing dentures, that prevents them from achieving a good seal. - Not wear headphones, jewelry, or other articles that may interfere with the seal. |  |  |  |  |  |
|  | - Emergency Procedures (respirator malfunction) |  |  |  |  |  |  |
|  |  | Page 4 |  |  |  |  |  |
|  |  | For any malfunction of an N95 or similar respirator (e.g., breakthrough, facepiece leakage, or improperly working valve in valve models), the respirator wearer must inform his or her supervisor that the respirator no longer functions, go to a designated safe area (away from resident care areas), and follow proper decontamination procedures to maintain the respirator. The supervisor must ensure the employee’s respirator is repaired or is provided with a new respirator. |  |  |  |  |  |
|  | - Cleaning, Maintenance, and Training |  |  |  |  |  |  |
|  |  | Cleaning – When respirators are authorized to be reused, care must be taken. Transmission from contact with a contaminated respirator can be reduced by not reusing respirators known to be contaminated (i.e., having come in contact with infected bodily fluid). Employees should consider use of a face shield over the respirator to reduce surface contamination, clean and sanitize hands before putting on and after taking off the respirator, and use clean gloves when putting a respirator on and performing the seal check. All employees shall follow the applicable cleaning procedures set forth in the Respiratory Protection Standard: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.134AppB2. |  |  |  |  |  |
|  |  |  | Maintenance - Respirators are to be properly maintained at all times to ensure that they function properly and adequately protect the employee. - Maintenance involves a thorough visual inspection for cleanliness and defects. - Worn or deteriorated parts will be replaced prior to use. - No components will be replaced, or repairs made beyond those recommended by the manufacturer. The following checklist will be used when inspecting respirators: - Facepiece: - Cracks, tears, or holes - Facemask distortion - Cracked or loose lenses/face shield - Valves: - Residue or dirt - Cracks or tears in valve material - Head straps: - Breaks or tears - Broken buckles - Filters/Cartridges: - approval designation - gaskets - cracks or dents in housing - proper cartridge for hazard - Respirators must be stored in a clean, dry area, and in accordance with the manufacturer’s recommendations in original manufacturer’s packaging. |  |  |  |  |
|  |  |  | Page 5 |  |  |  |  |
|  |  |  | - Respirators that are damaged, crushed, and defective or have defective parts, must be taken out of service immediately and presented to the Program Administrator for evaluation. - If, during an inspection, an employee discovers a defect in a respirator, it should be brought to the attention of the supervisor. - Supervisors will give all defective respirators to the Program Administrator. The Program Administrator will decide whether to take the respirator out of service until it can be repaired; perform a simple fix on the spot such as replacing a headstrap; or dispose of the respirator due to an irreparable problem or defect. When a respirator is taken out of service, it must not be reused, and the employee must be given a replacement of the same make, model, and size. If the employee is not given a replacement of the same make, model and size, then the employee must be fit tested for a new make, model, and size available before being allowed to use the respirator in a care situation. |  |  |  |  |
|  |  |  | Training - The Program Administrator will provide training to respirator users and their supervisors on the Respiratory Protection Program and their responsibilities under it, and on the OSHA Respiratory Protection Standard. - Employees will be trained prior to using a respirator in the workplace. - The training must be comprehensive and understandable, and must recur annually, and more often if necessary. - As with any employee, supervisors must be trained prior to using a respirator in the workplace; they also should be trained prior to supervising workers who must wear respirators if the supervisors themselves do not use a respirator. - Supervisors will provide the basic information on respirators in Appendix D of the Respiratory Protection Standard (appended to this Program) to employees who are voluntarily wearing a respirator. - Supervisors will ensure that each employee using a respirator can demonstrate knowledge of at least the following: - Why the respirator is necessary and how improper fit, usage, or maintenance can compromise the protective effect of the respirator. - What the limitations and capabilities of the respirator are. - How to use the respirator effectively in emergency situations, including situations in which the respirator malfunctions. - How to inspect, put on and remove, use, and check the seals of the respirator. - What the procedures are for maintenance and storage of the respirator. - How to recognize medical signs and symptoms that may limit or prevent the effective use of respirators. - The general requirements of the Respiratory Protection Standard. Retraining shall be administered annually, and when the following situations occur: Page 6 - Changes in the workplace or the type of respirator renders previous training obsolete. - Inadequacies in the employee’s knowledge or use of the respirator indicates that the worker has not retained the requisite understanding or skill. - Any other situation arises in which retraining appears necessary to ensure safe respirator use. |  |  |  |  |
| - Program Evaluation |  |  |  |  |  |  |  |
|  | The Program Administrator will conduct periodic evaluations of the workplace to ensure that the provisions of this Program are being implemented and information for client care and employee training is documented. The evaluations will include regular consultations with employees who use respirators and their supervisors, site inspections, and a review of records. Program evaluations should include documentation of discussion points with employees. Problems identified will be noted and corrected by the Program Administrator. |  |  |  |  |  |  |
| - Documentation and Recordkeeping |  |  |  |  |  |  |  |
|  | The following documentation and recordkeeping will be maintained: - A written copy of this Program and the Respiratory Protection Standard is kept in the Program Administrator’s office and is available to all employees who wish to review it. - Copies of training materials will be maintained in the Program Administrator’s office. - Copies of fit test records will be maintained in the Program Administrator’s office. - Records will be updated as new employees are trained and as existing employees receive refresher training. - Medical evaluation and examination records. The Program Administrator will maintain copies of medical evaluation and examination records for all employees covered under the Program. These records will be treated as confidential and will remain with the Program Administrator in a locked and secure location with limited access. |  |  |  |  |  |  |
|  |  |  | Page 7 |  |  |  |  |
| Appendix D |  |  |  |  |  |  |  |
|  | OSHA FORM FOR VOLUNTARY RESPIRATOR USE Appendix D to Sec. 1910.134. Information for Employees Using Respirators When Not Required Under the Standard Respirators are an effective method of protection against designated hazards when properly selected and worn. Respirator use is encouraged, even when exposures are below the exposure limit, to provide an additional level of comfort and protection for workers. However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the worker. Sometimes, workers may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards. If your employer provides respirators for your voluntary use, or if you provide your own respirator, you need to take certain precautions to be sure that the respirator itself does not present a hazard. |  |  |  |  |  |  |
|  |  | You should do the following: - Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning and care, and warnings regarding the respirator’s limitations. - Choose respirators certified for use to protect against the contaminant of concern. NIOSH, the National Institute for Occupational Safety and Health of the U.S. Department of Health and Human Services, certifies respirators. A label or statement of certification should appear on the respirator or respirator packaging. It will tell you what the respirator is designed for and how much it will protect you. - Do not wear your respirator into atmospheres containing contaminants for which your respirator is not designed to protect against. For example, a respirator designed to filter dust particles will not protect you against gases, vapors, or very small solid particles of fumes or smoke. - Keep track of your respirator so you do not mistakenly use someone else's respirator. |  |  |  |  |  |

| Signature: |  | Date: | / / |  |
| --- | --- | --- | --- | --- |

Page 8

Exhibit 1

Employees Participating in Medical Surveillance

| Employee Name | Date of Hire |
| --- | --- |

Page 9